# Hardware

- Windows Computer

- USB-C to USB Cable

- 24182 – Cable Assembly, Power, Focusing

- 24184 – Cable Assembly, Focusing Interface, 6X

- FLIR Boson VPC Adapter

# Software/Firmware

- 6X-21214_Programming

- 6XT-21216_Programming

- FLIR Boson App

## FLIR Boson App Install (21216 Only)

If not already installed, the following steps need to be completed to install the FLIR Boson App on the user’s PC.

- Copy to a local directory the installation file bosonAPP_v3.0.0_x64drivers.msi:

\\172.25.0.194\Data\Part Database (things we sell)\SW-xxxx-xxxx -- 6x Firmware\FLIR Boson\GUI

- Run the bosonAPP_v3.0.0_x64drivers.msi installation file.

- Follow the wizard instructions to complete the installation.

# Sensor programming

This section covers the programming steps for both the thermal and multispectral variants of the 6X sensor.  Files specific to a variant are includes in their applicable sub-folder:

- 6X: 6X-21214_Programming

- 6XT: 6XT-21216_Programming

Section 3.1 below pertains only to the thermal 6X and should be done before the sensor is installed into the lens mount.

## Thermal Sensor Configuration (21216 Only)

- Connect the FLIR Boson VPC adapter to the back of the thermal sensor.

- Connect the USB cable between the PC and the VPC adapter.

- Open the FLIR Boson App.

- In the lower right corner of the window, click the “Port” dropdown and select the COM port for the thermal sensor.

  - If the thermal sensor is the only USB device used, there will only be one COM port.

  - The correct COM port can also be viewed in the device manager.

- The “Boson Link” status should show connected.

- Click the “Image Appearance” button on the left column of options.

- Click on the “Analog/CMOS” video controls section.

  - Under the video section, click the “CMOS” checkbox.

  - Under the configure section, set video source to “TLinear”. The Output should be “TLinear”.

  - Under the CMOS & USB telemetry section, turn “CMOS Enable” on.

  - Under the CMOS & USB telemetry section, set the “Mode” drop down to “Header”.

- Click on the “System” button on the column on the left column of options.

  - Under the “Configuration Controls” section, click “Save Power-On Defaults”.

- Disconnect the sensor and close the GUI.

- To verify that the correct changes were made, reconnect the sensor, reopen the FLIR Boson app, and ensure that all the settings changes remained.

## Programming – Sensor Assembly

- Remove the microSD card from the camera and put it into the computer.

- Copy the contents of folder firmware - sdcard to the microSD card.

- Eject the microSD card from the computer and put it inside the camera.

- Remove the rear cover of the camera (if present).

- Set the dip switches present on the Imager Baseboard CCA to the following positions:

| Dip Switch | Position |
| --- | --- |
| 1 | ON |
| 2 | OFF |
| 3 | ON |
| 4 | OFF |

- Connect the Power cable (24182 & 24184) between an outlet and the camera.

- Connect the USB-C to USB cable between the PC and the camera.

- Navigate to 192.168.42.1 on a web browser.

- Navigate to the “Update Firmware” tab on the left side of the webpage.

- Click in the “Firmware Update” field and select the firmware update file (where “X.X.X” is the release version) from the firmware – factory update folder:

  - 6X: 6x-factory-update_X.X.X-21214.swu

  - 6XT: 6x-thermal-factory-update_X.X.X-21216.swu

- Once the firmware update is completed (approx. 5 minutes) power down the camera.

- Remove the microSD card from the camera and place it in the computer.

- Delete all files from the microSD card.

- Copy folder configs/2121X-XX/firmware to the microSD card.  Select the 2121X-XX folder based on what camera model is being programmed.

Note: The OEM – Sentera GPS configuration is applied as the initial configuration so that camera operation and focusing can be accomplished without being connected to a gimbal.

- On the microSD card, open file firmware/hw_config.yaml, and update the serial number to the serial number of the camera (surrounded by single quotes).

- Eject the microSD card from the computer and place it in the camera.

- Set the dip switches present on the Imager Baseboard CCA to the following positions:

| Dip Switch | Position |
| --- | --- |
| 1 | ON |
| 2 | ON |
| 3 | OFF |
| 4 | ON |

- Re-install the rear cover of the camera.

- Power on the camera.

- Navigate to 192.168.42.1 on a web browser.

- Navigate to the “Configuration” tab on the left side of the webpage.

- Verify that the “OEM – Sentera GPS” configuration is being used.

- Navigate to the “Diagnostics” tab on the left side of the webpage.

- Verify that the Part Number and Serial Number are what is expected for the camera.

- Programming is complete!